| SUBJECT/TITLE: | Safety Procedures for Use of Swing Stage |
| --- | --- |
| PURPOSE: | To ensure safe working conditions for employees using or setting up swing stage. |

A. In order to ensure personnel safety, these guidelines are to be followed during the set-up 	and/or use of the swing stage.

1. Before each use, these and all operating rules shall be reviewed with the 	 	 personnel using this equipment by the Manager of the Craft Division of 	  	 Engineering Services, or his/her designee.

a. Read the operating and maintenance instructions.  Follow them at all 	times.

b. Be sure roof or supporting structure is strong enough to support 	hoist and rigging equipment.  If any question exists, ask.

c. Inspect all equipment and make sure all bolted connections are 					tight.

d. Make sure all outriggers are properly tied back.  (See instructions.)

e. Use only specially designed counter weights and make sure they are 	fastened in the proper place on the outrigger.  (See instructions.)

f. Make sure the cables are fastened properly and are long enough.  	          	They should reach from the top to the lowest point plus ten feet for 	 rewinding.

g. CAUTION:  Do not set up swing stage in the vicinity of power      	lines.

h.	Never get on the stage without a fall arrest device that is secured 		properly to a secured device separate from the stage and 	outriggers.

i. Inspect all cables and riggings before every use.

j. 	Check electric supply and cords before every use.

k. Do not put a heavier load on the stage than is recommended.

l. Check all railings and toe kicks to be sure they are securely in place 			before every use.

m. When loading items on to the stage, set them down gently.  Avoid 	  	 any and all sudden strains.

Source: UI Healthcare Engineering Services

Date effective: July1, 1995

Date Revised:

Date Reviewed: April 6, 2016