Policy Statement B-212

Attachment

Machine Guards

All machine guards should:

A.	Be considered a permanent part of the machine or 					equipment, while, at the same time, be easy to remove or 				replace.

B.	Prevent access to the danger zone while the machine is 				operating.

C.	Be convenient so that they do not interfere with efficient 				operation or maintenance of the machine, nor should they 				greatly inconvenience the operator.

Any rotating part of a machine must be guarded against accidental 	contact.  The rotating mechanism should be evaluated to determine 		whether a complete or partial enclosure is necessary.

Flywheels should be guarded against with a railing or with an enclosure of 	sheet metal, perforated or expanded metal, or woven wire.

Exposed parts of horizontal, vertical or inclined shafts seven feet or less 		from the floor or work platform should be enclosed with a stationary 	casing.

Gears should be enclosed on all sides with no opening exceeding one-	half inch, if the guard is within four inches of the gear.  Gear guards 	should be made of metal.

Fans lower than seven feet from the floor should be protected with a mesh 	guard completely covering the blades.

Cutters, band and circular saws, and abrasive wheels are typical of those 	machines that present a cutting or shearing hazard.  Personnel should 	be careful when using these items, even when the machine has guarding 	equipment.

Circular saws should be guarded by a hood that will cover the teeth at 	all times to at least their depth.  Such hoods should self-adjust 	automatically to the thickness of, and remain in contact with, the material 	being cut.

Table saws should be equipped with a spreader, splitter, or riving knife to 	keep material from the back edge of the saws.  Material kicked back from 	table or radial arm saws is frequently the cause of serious injury.  Injuries 	also come from accidentally placing the hands and fingers against the 	knives.  A high percentage of accidents occur in the working of short 	lengths of stock.

Jointers should have cylindrical heads.  The throat in the cylinder should 	not be deeper than 7/16 inch, nor wider that 5/8 inch.  The openings 	between the table and the head should be just large enough to clear the 	knife.  Cuts heavier than recommended by the manufacturer should be 	avoided because of the danger of kickbacks.

A guard that adjusts itself as the stock strikes against it should cover the 	table opening on the working side of the cage.

Wood shapers are dangerous in that hands may strike against the knives.  	Serious accidents also result from broken knives being thrown by the 	machine. Shapers should have guards that cover the top of the spindle 	and the knives and which surround the knives just high enough to clear 	the stock.  The use of jigs is essential to fully protect the hands.  As with 	the jointer, deep cuts should be avoided to reduce the possibility of 	kickbacks.

The ends of belt sanding machines should be enclosed with metal guards 	that also serve as part of the exhaust system.

Injury in the use of lathes comes from contact with projections on face 	plates and chucks and from contact with lathe dogs with projecting set 	screws.  Flying metal chips, hand braking of the machine, filing right-	handed, using the hand instead of a stick to hold emery cloth against the 	work, calipering or gauging the job while the machine is in operation, all 	represent hazardous conditions or practices and should be avoided.  	Lathe operators should wear eye protection equipment when using lathes.

Grinding and buffing wheels should have exhaust hoods to catch particles 	thrown off by the wheels.  Eyes should be protected with a fixed 	transparent shield on the machine or with proper eyewear (goggles or 	safety glasses).  Sufficient lighting should be provided to the point of 	contact on the grinder so that the operator can remain a reasonable 	distance from the wheel.  The tool rest should be not more than 1/8 inch 	from the wheel itself.  Grinders require care in the selection of wheels, 	mounting, flanges, adjustment and use.

Soldering

The following safety measures must be taken when soldering:

A.	Have both water and carbon dioxide fire extinguishers at the 			job location.

B.	Wear goggles or face shield, gloves, and, when necessary, 				arm protectors.

C.	Keep sleeves rolled down, shirt collar buttoned, and trouser 				legs over the shoe tops.

D.	Melt solder only in a thoroughly dry pot or ladle which is 				secured so it cannot be upset.

E.	Never put chilled solder or a moist object into a hot solder 				pot or ladle.

F.	Place hot irons on racks or holders away from all 					combustible materials.

G.	Do not test the temperature of an iron by holding it near the 				face.

H.	Do not snap or throw solder to remove the excess from a hot 			iron.

I.	Make sure all explosive vapors have been removed from 				containers to be soldered.

J.	Avoid igniting material surrounding the area being soldered.

Welding and Cutting

Welding and cutting operations present extreme hazards to the operators, 	individuals in the surrounding area, and nearby materials.  Therefore, all 		measures listed below must be taken when welding or cutting.

A.	Have both water and carbon dioxide fire extinguishers at the 			job location.

B.	Wear welding helmet, gloves, and arm shields.

C.	Keep sleeved rolled down, shirt collar buttoned, and trouser 				legs over the shoe tops.

D.	Survey the area for flammable material.  Welding or cutting 				should not be done in or near rooms containing liquids, 				vapor or dust, nor on tanks that may still contain 					flammable vapors.

E.	Protect flammable material in welding area with flame 				resistant canvas or sheet metal.

F.	Inspect welding or cutting equipment for safety before 				operating.

G.	Remove combustible coverings or deposits from surfaces 				that are to be welded or cut.

H.	Avoid the creation of dust or flammable vapors during 				welding operations.

I.	Keep area surrounding welding or cutting operations under 				fire surveillance for at least one-half hour after welding or 				cutting has been completed.

Torches

Gasoline, kerosene, or alcohol torches should be placed and used to that 	their flames are at least 18 inches from wood surfaces.  They should not 	be used in the presence of dusts or vapors, or near flammable liquids, 	paper, excelsior, or similar material.  Torches should not be left 	unattended while they are burning.

Apparel

Hospital uniforms are to be worn by Facility Operations - Maintenance and 	Engineering personnel.  Shirts must be worn with tails tucked in to avoid 	the hazard of catching them in moving machinery.  Poorly fitting garments 	are to be exchanged for ones of the correct size.

Clothing that becomes soaked in oil for flammable solvent should be 	changed immediately since it constitutes a definite fire hazard to the 	wearer, while also presenting the possibility of skin irritation.

The wearing of rings, bracelets, and wristwatches can result in serious 	injuries.  A finger may be torn completely away if a ring catches on a 	moving machine part or on a fixed object when the body is moving rapidly.  	Metal watchbands worn near electrical equipment can be dangerous, 	since metals are good conductors of electricity.  For these reasons, 	maintenance employees must take care to remove rings, watches, and 	other hazardous jewelry when performing tasks in which these items 	present a hazard.

Portable Power Tools

The manufacturer’s instructions for use and maintenance of portable 	power tools should be followed carefully by those using these items.  An 	employee who must use a certain portable power tool should be able to 	inspect the tool for unsafe conditions.

Safety glasses or face shields should be worn by users of portable power 	equipment that generate chips or dust, or which are subject to 	breakage.  Gloves, ties, jewelry, and loose clothing are not to be worn by 	those using drills, saws, or grinders.

Users of power equipment should always turn off the power source before 	accessories are connected or disconnected.  Machine guards, if removed, 	should be replaced before operating the machinery again.

Frequent inspections of power equipment should be made to spot worn or 	broken external wiring in time to prevent injury.  However, no matter how 	good the condition of a machine’s wiring, the use of portable electric tools 	in wet locations is not recommended.

All portable electric tools should be grounded.  This is to be considered a 	condition of their use.  To guarantee a grounded piece of equipment, the 	user should insist that a three-pronged plug be present on the equipment 	cord.  The three-pronged plug should only be inserted in a three-pronged 	receptacle.

Portable Electric Drills

Electric drills can cause injuries in several ways.  The drill may be pushed 	into the hand, the leg, or other parts of the body, or dropped on the feet 	when the operator is not drilling.  Eyes may be hit by material being drilled 	or by parts of a broken drill.

Selection of drill bits no longer than necessary will give some protection.  	Oversized bits should not be ground down to fit small drills.  An adapter 	should be used.  Small pieces of work should be clamped or anchored to 	prevent their movement.

Portable Power Saws

Guards on portable electric saws should be checked for their proper 	functioning.  These saws should not be jammed or crowded into the work.  	Saws should be started and stopped outside the work and the operator 		should keep his body out of the line of cut as much as he can.

Portable Electric Grinding Wheels, Buffers, and Wire Brushes

On these pieces of equipment, the exposed part of the wheel should be 	no wider than is needed to reach the work.  In no case should it exceed 	180 degrees.  Operators should wear safety glasses or face shields.

Sanders

The operator should keep the sander away from his body and his clothes.  	Dust goggles should be worn and, if harmful dusts are created by the 	sander, a respirator should be worn.  To assure safe operation, sanders 	should be cleaned regularly.  Portable sanders create the same fire and 	explosion hazards as fixed sanding equipment; therefore, adequate 	ventilation should be provided.

Ladders

The proper use of ladders is important to the safety of all personnel.  The following points should be noted:

A.	The feet of the ladder should be suited to floor surface.

B.	Ladders should be of correct length, strength, and condition.  			Any ladders with an obvious structural defect are to be 				discarded.

C.	Ladders should be positioned correctly.

D.	An individual using a ladder should remain clear of moving 				parts, electrical conductors, steam pipes and other similar 				hazards.