| SUBJECT/TITLE: | Operating a Kubota RTV |
| --- | --- |
| PURPOSE: | To facilitate safe operation of a Kubota RTV. |

I. The Engineering Services Department will provide proper training to staff that operate any Kubota RTV Utility Vehicle.

II. Required PPE:

- Appropriate Gloves as needed 		4. Safety Shoes/ Boots with Reinforced Toe

- Hard Hat as needed				5. High Visibility Vest as needed

- Safety Glasses as needed			6. Dust Mask as needed

III. Required Training:

- Read, understand, and follow manufacturer’s safety instructions.

- Training with qualified person prior to use.

| TASK | HAZARDS | CONTROLS |
| --- | --- | --- |
| Preparing Job Site And Equipment | Trips | Clear area within work zone; Remove trip hazards and/ or mark clearly hazards with cones, etc. |
|  | Cuts | Wear gloves when checking tires |
|  | Exposure to Oil/Coolant | Wear safety glasses and gloves if adding fluids |
|  | Bodily Injury to Others | Identify work zone with cones, barricades and other means necessary to keep pedestrian and other traffic out of the work zone |
| Operational Considerations | Bodily Injury | Wear proper PPE as needed for each assigned job duty |
|  |  | A pre-job discussion should occur to ensure both the equipment operator and assisting workers understand the scope of the project |
|  |  | Operator should keep watch for ground workers near equipment and ensure they are aware of operator's intended direction of movement. Use spotter as need to warn/watch for ground workers. |
|  |  | Ground workers should watch operator and equipment, staying clear of equipment's path |
|  |  | All workers need to be aware of changing conditions at work site area. |
| Equipment Operations | Falling Off Equipment | Fasten seatbelt properly as needed |
| Equipment Operations | Rollover | Do not drive recklessly |
|  |  | Do not make hard fast turns |
|  |  | Operate within manufacturer listed stability speeds |
|  |  | Keep loads balanced and secured Do not carry more than one passenger in protected cab |
|  | Bodily Injury to Operator | Never reach into areas with moving parts such as tires, hydraulics arms, and engine area while engaged |
|  |  | Never leave equipment running to clear tires or troubleshoot problems |
|  |  | Do not operate damaged or malfunctioning equipment |
|  |  | Be aware of changing conditions at worksite |
|  |  | Wear as needed PPE for each job accordingly |
| After operations or during periods when area is not occupied by workers | Bodily Injury to Workers and Others | Operator should leave bed in lowered position |
|  |  | Operator should ensure equipment is secured by pulling the key and set parking brake |
|  |  | Workers should secure job site with barricades, cones and signs to warn others to keep out of work site area. |
|  |  | Supervisor may place employee to watch job site if extreme hazards exist. |
| Servicing or Repair of Equipment | Bodily Injury | Obtain tag from shop, mark equipment "Do Not Use"- Specify problem |
|  |  | Notify shop and/or take equipment to shop |

Training: Operating A Kubota RTV Utility Vehicle

Training Video: https://www.youtube.com/watch?v=vTrYIap9KQo

DO NOT use this machine unless you have been trained in its safe use and operation. Training must be documented and signed by the employee before use.

| Description of Work: |  |  | Operating A Kubota RTV Utility Vehicle |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Potential Hazards: Exposed moving parts with the potential to cause harm through pinching, impact and crushing, shearing injuries, ergonomics, vibration, noise, slips, trips and falls and dust. |  |  |  |  |  |
| Personal Protective Equipment (PPE) Required |  |  |  |  |  |  |  |  |  |  |
| Gloves |  | Face Masks |  | - Eye - Protection |  | Welding Mask | Appropriate Footwear |  | Hearing Protection | Protective Clothing |
| Safe Work Procedure Checklist: |  |  |  |  |  |  |  |  |  |  |
| 1. PRE-Operation: - Check for obvious faults and defects including fuel/hydraulic leaks or damage. - Identify ON/OFF key switch and parking brake - Wear adequate and appropriate PPE. 2. Operation: - Use of seat belt as needed. - Keep hands and feet inside unit. - Adjust view mirrors accordingly. - Use both hands on steering controls. - Ensure firm footing. - Always keep eyes on direction of travel. - Use horn when approaching blind corners. - Do not have more than one passenger in cab. - Do not keep unit in four wheel drive: Unless needed. - Know the acceptable amount of load unit can safely handle and secure load as needed. - Keep bed in lowered position; always load/unload on flat terrain. - Use smooth operation when shifting and accelerating. 3. POST-Operation: - Ensure no damage has been made to unit or fluid leaks. - Ensure unit is parked on flat terrain and parking brake set. - Ensure equipment is returned clean to storage area after use. |  |  |  |  |  |  |  |  |  |  |
| Competent Persons (The following persons are authorised to operate equipment.) |  |  |  |  |  |  |  |  |  |  |
| Date: | Name: |  |  |  |  |  |  | Trainer Signature: |  |  |

Source: UI Healthcare Engineering Services

Date effective: July 1, 2016

Date Reviewed: July 1, 2016