| SUBJECT/TITLE: | Operating a Gas Powered Pressure Washer |
| --- | --- |
| PURPOSE: | Safe Operation of a gas powered washer. |

Policy Statement:

The Engineering Services Department will provide proper training to staff operating a gas powered pressure washer.

Modification to Daily Attire Requirements: No exceptions will be allowed unless approved by the Director

Required PPE:

- Safety Shoes/ Boots with Reinforced Toe	4. Safety googles or face shield

- Appropriate gloves as needed 		5. Long pants

- Hearing Protection, Ear Plugs, or Muffs

Required Training:

- Read, understand, and follow manufacturer’s safety instructions

- Training with qualified person prior to use

| TASK | HAZARDS | CONTROLS |
| --- | --- | --- |
| Preparing Pressure Washer for Use | Exposure to Fuel Mix: Skin & Eyes | Wear appropriate chemical resistant gloves and safety glasses or face shield. |
|  | Fire/Explosion | Never fuel equipment indoors; keep fuel away from open flames or sparks |
|  |  | Clean up fuel spill immediately; do not start engine until fumes have dispersed. |
|  |  | Dispose of spilled fuel per SPCC Plan training: contain, label for disposal, and contact EHS |
| Operating Pressure Washer | Muscle Strain | Pull cord straight back; provide for clearance of arm and elbow. |
|  |  | With wand use short, sweeping motions, avoid twisting and bending repetitively. |
|  |  | Use safe lifting training-back injury prevention training. Wear appropriate gloves to minimize vibration and fatigue on hands |
|  | Hearing Damage | Wear earplugs or earmuff headset |
|  | Inhalation of Dust/particulates by Self and or Others | Avoid creating excessive dust - pre-wet or wet area as needed. Pause work to allow pedestrians-bicyclists or motorists to pass. |
|  | Eye or Other injury to Self and or Others | Wear safety glasses or goggles. |
|  |  | Pause work to allow pedestrians-bicyclists or motorists to pass. Never wash towards pedestrians! |
|  |  | Set up safety cones or barricades, as needed, if work affects areas with pedestrian, etc. |
| Operating Pressure Washer | Insect bites | Wear long pants or equivalent protection |
|  |  | Be aware of potential ground bees or yellow jacket nests. |
| Storage/transporting | Fire | Do not place hot equipment on or neat combustible materials or flammable liquids. |
|  |  | Allow engine to cool before storing - avoid contact with combustibles. |
|  |  | Store properly to prevent fluids leaking out of equipment. |
|  |  | Always secure equipment when in transit. |
| Procedure for malfunction or broken equipment | Bodily Injury | Obtain tag and mark equipment "Do not use" - specify problem or hazard, date, name of writer. |
|  |  | Notify grounds supervisor immediately and take equipment to shop. |

Training: Operating A Gas Powered Pressure Washer

Training Video: http://www.mitm.com/support/videos/how-to-start-a-gas-pressure-washer/

DO NOT use this machine unless you have been trained in its safe use and operation. Training must be documented and signed by the employee before use.

| Description of Work: |  |  | Operating A Gas Powered Pressure Washer |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Potential Hazards: Potential to cause harm through forced water, impact and abrasion, exhaust heat or crushing injuries, ergonomics, vibration, noise, slips, trips and falls and dust. |  |  |  |  |
| Personal Protective Equipment (PPE) Required |  |  |  |  |  |  |  |  |  |  |
| Gloves |  | Face Masks |  | - Eye - Protection | Welding Mask |  | Appropriate Footwear |  | Hearing Protection | Protective Clothing |
| Safe Work Procedure Checklist: |  |  |  |  |  |  |  |  |  |  |
| 1. PRE-Operation: - Check for obvious faults and defects including fuel leaks or damage. - Identify ON/OFF switch. - Wear adequate and appropriate PPE. 2. Operation: - Exercise caution in relation to exhaust heat. - Hold pressure wand firmly. - Ensure firm footing. - Move your body with the direction of work; do not twist from the waist. - Use trip hazard signs or cones. - Do not spray towards pedestrians or traffic. 3. POST-Operation: - Ensure pressure washer engine has stopped and water off before releasing pressure on hose couplings. - Ensure pressure hoses are drained of water and couplers hooked together and wand tips are placed back in holder. - Ensure equipment is returned to storage area after use and full of fuel. |  |  |  |  |  |  |  |  |  |  |
| Competent Persons (The following persons are authorised to operate equipment. |  |  |  |  |  |  |  |  |  |  |
| Date: | Name: |  |  |  |  |  |  | Trainer Signature: |  |  |

Source: UI Healthcare Engineering Services

Date effective: April 6, 2016

Date Reviewed: June 30, 2016