| SUBJECT/TITLE: | Operating a Gas Powered Weed Trimmer |
| --- | --- |
| PURPOSE: | Safe Operation of a gas powered weed trimmer. |

Policy Statement:

The Engineering Services Department will provide proper training to staff operating a gas powered weed trimmer.

Modification to Daily Attire Requirements: No exceptions will be allowed unless approved by the Director

Required PPE:

- Safety Shoes/ Boots with Reinforced Toe	5. Safety Glasses or Face Shield

- Appropriate Gloves as needed 		6. Appropriate Clothing: Long Pants

- Hearing Protection, Ear Plugs, or Muffs	7. Dust Mask as needed

- Reflective Vest as needed

Required Training:

- Read, understand, and follow manufacturer’s safety instructions

- Training with qualified person prior to use

| TASK | HAZARDS | CONTROLS |
| --- | --- | --- |
| Preparing / Fueling Equipment | Skin or Eye Exposure to Fuel / Oils | Wear appropriate gloves for fuel |
|  |  | Wear safety glasses |
|  | Fire | Never add fuel to hot or running equipment |
|  |  | Always add fuels outdoors, ensure fuel cap is secure before use of equipment |
|  |  | Do not add fuel near open flame, sparks or other ignition sources. |
|  |  | Clean fuel spills immediately; SPCC plan. Do not start engine until fumes have dispersed. |
| Operating Trimmer | Eye Injury | Wear safety glasses or full face shield to prevent injury from flying objects. |
|  | Hearing Damage | Wear hearing protection: ear plugs or ear muffs. |
|  | Bodily Injury: Cuts, Scrapes, Burns, etc. | Remain aware of your surroundings and be aware of changing conditions. |
|  |  | Watch for loose materials that can become airborne. |
|  |  | Maintain control of equipment at all times. |
|  |  | Keep hands and fingers away from moving parts. |
|  |  | Avoid contact with hot engine parts. |
|  | Muscle Strain | Use proper ergonomics; maintain balance, avoid overreaching and twisting. |
|  |  | Never bend and twist at the same time. |
| Operating Trimmer | Injury to Others | Use signs, cones, and barricades to warn others of work activities when in high traffic areas. |
|  |  | Maintain a reasonable safety zone from others - do not allow debris to fly towards pedestrians or vehicles. |
|  |  | Suspend use if heavy pedestrian traffic is present |
|  |  | Be aware of changing conditions. |
| Site Safety | Bodily Injury from Vehicles, Bicyclists, Etc. | Place signs, cones, barricades, caution tape, and lookouts as needed. |
|  |  | Wear reflective safety vests when in streets |
| Procedure for malfunctioning or broken equipment | Bodily Injury | Obtain tag and mark equipment "Do not use"- specify problem or hazard, date, name of writer. |
|  |  | Notify grounds supervisor immediately and take equipment to shop. |

Training: Operation Of Gas Weed Trimmer

Training Video: https://www.youtube.com/watch?v=1Dt5C1vA5QA

DO NOT use this machine unless you have been trained in its safe use and operation. Training must be documented and signed by the employee before use.

| Description of Work: |  |  | Operating A Gas Powered Weed Trimmer |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Potential Hazards: Potential to cause harm through flying debris, impact and abrasion, exhaust heat or crushing injuries, ergonomics, vibration, noise, slips, trips and falls and dust. |  |  |  |  |
| Personal Protective Equipment (PPE) Required |  |  |  |  |  |  |  |  |  |  |
| Gloves |  | Face Masks |  | #### Eye #### Protection | Welding Mask |  | Appropriate Footwear |  | Hearing Protection | Protective Clothing |
| Safe Work Procedure Checklist: |  |  |  |  |  |  |  |  |  |  |
| 1. PRE-Operation: - Check for obvious faults and defects including fuel leaks or damage. - Identify ON/OFF switch. - Wear adequate and appropriate PPE. 2. Operation: - Exercise caution in relation to exhaust heat. - Hold trimmer firmly. - Ensure firm footing. - Move your body with the trimmer; do not twist from the waist. - Do not weed trim towards pedestrians. Move from right to left when cutting. 3. POST-Operation: - Ensure weed trimmer engine have stopped before removing any debris under guard. - Ensure engine has stopped before placing trimmer on the ground. - Ensure equipment is returned to storage area after use. |  |  |  |  |  |  |  |  |  |  |
| Competent Persons (The following persons are authorised to operate equipment. |  |  |  |  |  |  |  |  |  |  |
| Date: | Name: |  |  |  |  |  |  | Trainer Signature: |  |  |

Source: UI Healthcare Engineering Services

Date effective: July 7, 2016

Date Reviewed: August 3, 2016