| SUBJECT/TITLE: | Operating a Kubota Riding Mower |
| --- | --- |
| PURPOSE: | Safe Operation of Kubota Riding Mower. |

Policy Statement:

The Engineering Services Department will provide proper training to staff that operate any Kubota Riding Mower.

Required PPE:

- Safety Glasses				4.   Safety Shoes/ Boots with Reinforced Toe

- Dust Mask as needed			5.   Hearing Protection, Ear Plugs, or Muffs

- High Visibility Vest as needed

Required Training:

- Read, understand, and follow manufacturer’s safety instructions

- Training with qualified person prior to use

| TASK | HAZARDS | CONTROLS |
| --- | --- | --- |
| Preparing Job Site And Equipment | Trips | Clear area within work zone; Remove trip hazards and/ or mark clearly hazards with cones, remove large branches, etc. |
|  | Cuts | Wear gloves when checking tires and attaching implements. |
|  | Exposure to Oil/Coolant | Wear safety glasses and gloves if adding any fluids. |
|  | Bodily Injury to Others | Identify work zone if needed use cones, barricades and other means necessary to keep pedestrian and other traffic out of the work zone. |
| Operational Considerations | Bodily Injury | Wear reflective safety vest when traveling on roadways. |
|  |  | A pre-job discussion should occur to ensure both the equipment operator and assisting workers understand the scope of the project when needed. |
|  |  | Operator should keep watch for ground workers near equipment and ensure they are aware of operator's intended direction of movement. Use spotter as need to warn/watch for ground workers. |
|  |  | Ground workers should watch operator and equipment, staying clear of equipment's path and mower discharge. |
|  |  | All workers need to be aware of changing conditions at work site area. |
| Equipment Operations | Falling Off Equipment | Ensure firm footing and use of handle grips as needed |
| Equipment Operations | Rollover | Do not turn sharply on hill sides at high speeds |
|  |  | Do not load/unload riding tractor on uneven ground |
|  |  | Operate within manufacturer listed stability speeds |
|  | Bodily Injury to Operator | Never reach into areas with moving parts such as tires, mower deck belts, PTO attachments. |
|  |  | Never leave equipment running to clear tires, debris from belts or troubleshoot problems |
|  |  | Do not operate damaged or malfunctioning equipment. |
|  |  | Be aware of changing conditions at worksite. |
|  |  | Be aware of fluid leaks. |
| After operations or during periods when area is not occupied by workers | Bodily Injury to Workers and Others | Operator should always leave equipment with PTO disengaged and turned off. |
|  |  | Operator should ensure equipment is secured/locked out so cannot be used by unauthorized personnel. |
|  |  | Workers should secure job site if needed with barricades, cones and signs to warn others to keep out of work site area. |
|  |  | Supervisor may place employee to watch job site if extreme hazards exist. |
| Servicing or Repair of Equipment | Bodily Injury | Obtain tag from shop, mark equipment "Do Not Use"- Specify problem |
|  |  | Notify shop and/or take equipment to shop |

Training: Operating a Kubota Riding Mower

Training Video: https://www.youtube.com/watch?v=zgDadsKffRw

DO NOT use this machine unless you have been trained in its safe use and operation. Training must be documented and signed by the employee before use.

| Description of Work: |  |  | Operating A Kubota Riding Mower |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Potential Hazards: Exposed moving parts with the potential to cause harm through pinching, impact and crushing, ergonomics, vibration, noise, slips, trips and falls and dust. |  |  |  |  |
| Personal Protective Equipment (PPE) Required |  |  |  |  |  |  |  |  |  |  |
| Gloves |  | Face Masks |  | - Eye - Protection | Welding Mask |  | Appropriate Footwear |  | Hearing Protection | Protective Clothing |
| Safe Work Procedure Checklist: |  |  |  |  |  |  |  |  |  |  |
| 1. PRE-Operation: - Check for obvious faults and defects including fuel leaks or damage. - Identify ON/OFF key switch. - Wear adequate and appropriate PPE. 2. Operation: - Always be looking ahead for obstacles. - Watch for trees, buildings, overhangs and other obstructions. Allow enough room to safely pass between obstructions. Be careful not to get too close. - Check overhead clearance before passing beneath branches, awnings and other obstructions. - Keep hands and feet on unit. - Ensure firm footing. - Always keep eyes on direction of travel. - Do not have riders. - Do not allow people to approach when PTO is engaged. - Know the limits of the machine. - Use smooth operation on controls. - Engage PTO at low throttle. 3. POST-Operation: - Ensure no damage has been made to unit or fluid leaks. - Ensure unit is parked on flat terrain and parking brake set. - Ensure equipment is returned clean to storage area after use. |  |  |  |  |  |  |  |  |  |  |
| Competent Persons (The following persons are authorised to operate equipment.) |  |  |  |  |  |  |  |  |  |  |
| Date: | Name: |  |  |  |  |  |  | Trainer Signature: |  |  |

Source: UI Healthcare Engineering Services

Date effective: October 12, 2016

Date Reviewed: