| SUBJECT/TITLE: | Operating a Gas Powered Pole Saw |
| --- | --- |
| PURPOSE: | Safe Operation of a Gas Powered Pole Saw. |

Policy Statement:

The Engineering Services Department will provide proper training to staff that operate any Gas Powered Pole Saw.

Required PPE:

- Safety Shoes/ Boots with Reinforced Toe	5.   Safety Googles or Face Shield

- Appropriate gloves				6.   Chaps

- Hearing Protection, Ear Plugs or Muffs	7.   Hard Hat

- Safety Vest					8.   Dust Mask as needed

Required Training:

- Read, understand, and follow manufacturer’s safety instructions

- Training with qualified person prior to use

| TASK | HAZARDS | CONTROLS |
| --- | --- | --- |
| Preparing Saw | Hand Injuries | Wear leather (or other durable) gloves, use caution when handling blades. |
| Fueling/Oiling | Fire | Always add fuel outdoors, ensure fuel cap is secure before use. |
|  |  | Do not add fuel if near open flame or sparks. |
|  |  | Clean-up fuel spills immediately; do not start engine until fumes have dispersed. |
|  | Chemical Exposure: Skin or Eyes | Wear appropriate gloves and eye protection when fueling. |
| General Use | Bodily Injury | Wear all required PPE. Use of cones or barricades to maintain safe work zone. |
|  |  | Keep all shields in place when not in use. |
|  |  | Keep hands, clothing, and hair clear of all moving parts. |
|  |  | When working overhead or under other workers, wear hard hat. |
|  | Inhalation of Dusts or Particles from Cutting | Wear dust mask as needed. |
|  | Eye Injury | Wear safety goggles or face shield to prevent eye/face injury from flying wood chips or vegetation, etc. |
|  | Hearing Damage | Wear hearing protection: ear plugs or earmuffs. |
|  | Foot Injury | Wear safety shoes or boots with reinforced toe. |
|  | Injury to Others | Use cones, barricades or other department staff members to keep others clear of area. Staff members may need to wear PPE. |
| General Use | Bodily Injury from Vehicles, Bicyclists, etc. | Place signs, cones, barricades, caution tape and lookouts as needed. |
|  |  | Wear reflective safety vests. |
| Additional Precautions for Fuel Powered Engines | Injury and Fire | Always operate in well-ventilated area. |
|  |  | Never fuel engine while running. |
|  |  | Stop engine before performing maintenance or trouble shooting. |
| Procedure for Malfunctioning or Broken Equipment | Bodily Injury | Obtain tag from shop, mark equipment "Do Not Use"- Specify problem. |
|  |  | Notify shop and/or take equipment to shop. |

Training: Operating a Gas Powered Pole Saw

Training Video: https://www.youtube.com/watch?v=b3AD1dbNd78

DO NOT use this machine unless you have been trained in its safe use and operation. Training must be documented and signed by the employee before use.

| Description of Work: |  |  | Operating A Gas Powered Pole Saw |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Potential Hazards: Potential to cause harm from chain, impact and abrasion, exhaust heat or crushing injuries, ergonomics, vibration, noise, slips, trips and falls and dust. |  |  |  |  |
| Personal Protective Equipment (PPE) Required |  |  |  |  |  |  |  |  |  |  |
| Gloves |  | Face Masks |  | Eye Protection | Welding Mask |  | Appropriate Footwear |  | Hearing Protection | Protective Clothing |
| Safe Work Procedure Checklist: |  |  |  |  |  |  |  |  |  |  |
| 1. PRE-Operation: - Check for obvious faults and defects including fuel leaks or damage. - Identify ON/OFF switch. - Wear adequate and appropriate PPE. 2. Operation: - Exercise caution in relation to exhaust heat. - Hold firmly with both hands. - Ensure firm footing. - Move your body with the direction of work; do not twist from the waist. - Use trip hazard signs or cones when in operation. - Do not allow pedestrians to travel in work area. 3. POST-Operation: - Ensure pole saw engine has stopped and cover is over chain. - Ensure pole saw is clean from dirt and debris. - Ensure equipment is returned to storage area after use and full of fuel and bar oil. |  |  |  |  |  |  |  |  |  |  |
| Competent Persons (The following persons are authorised to operate equipment. |  |  |  |  |  |  |  |  |  |  |
| Date: | Name: |  |  |  |  |  |  | Trainer Signature: |  |  |

Source: UI Healthcare Engineering Services

Date effective: June 6, 2016

Date Reviewed: