| SUBJECT/TITLE: | Failure of Deep Well Pump |
| --- | --- |
| PURPOSE: | To maintain operation of deep well pump. |

A. PROCEDURE:

1. If the Deep Well pump fails, back up can be supplied with city water.

a. Shut off Deep Well pump main breaker located by the pump in

B10-A2.

b. Close ball valve (V1), located above B10-A3.

c. Open city water valve, (V2), located at back flow preventor just north of 		     V1.  If there is no flow through V2, close V2 and open V3, located 			     above B-13 to feed city water.

d. Reverse the above process to switch back to the Deep Well pump from city 		     water.

Related and Supporting Documentation

•	Utilities Management Program, Engineering Services Policy C-001

•	Safety Reference Cards

•	On Call and Call Back Procedures, Engineering Services Policy A-115

Date effective: July 1, 1995

Date Reviewed: April 21, 2016