| SUBJECT/TITLE: | Access to JPP P.E.T. Secured Roof Areas |
| --- | --- |
| PURPOSE: | To properly access PET roof area for repair work. |

The Cyclotron in P.E.T. will automatically shut down if unauthorized entry occurs through any armed door or hatch.

A. PROCEDURE:

1.	Time of day access to roof:

a. During normal P.E.T. operating hours (8:00 A.M. until 6:00 P.M.), 			    schedule planned rooftop access prior to required date and time with 			    P.E.T. personnel. (Phone number 356-4100.)  The cyclotron can not 			    be operated when the security system is disarmed.

b. After hours access to roof (6:00 P.M. until 8:00 A.M.).

Contact Safety and Security (356-2658) prior to roof access.  They will 			    observe the following procedures for roof access instead of P.E.T. 			    personnel.

2.	Each roof door or hatch has a specific security access.  Determine the 			particular door or hatch that is required to access the area on the roof where 		the work will be performed.

3.	Under P.E.T. supervision (See 1A), or Security (See 1B), proceed to power 		supply room (JPP-0825) in the P.E.T. Imaging Center.  Record user access 		into security log, rotate specific key in security panel from arm to disarm 			position.  Remove key from security panel.  Key can not be removed in the 		Alarm position.

a. This room has restricted access and requires P.E.T. personnel or 				Safety and Security personnel to be present in order to access the 				security panel.

b. Security Panel reports the status of the key as either:

1. In the Security Panel

2. In transit (I.E., to or from roof)

3. In a specific roof top access panel

4.	Proceed directly to the specific roof top location and disarm door or hatch 			alarm by inserting key and rotating to disarm position.  Key can not be 			removed in the disarm position.

5.	Enter roof area and complete required work.

6.	Exit room area by same door or hatch as entered.  Failure to do so will 			activate the security alarm in the P.E.T. area and at the roof top door or 			hatch.  The key for the specific door is required to reset these alarms.

7.	Close door or hatch and turn key from disarm to reset to arm position.  			Proceed immediately to the P.E.T. area.

8.	Under P.E.T. supervision (see 1A) or Safety and Security (see 1B), return key 		to security panel in power supply room (JPP-0825).  Rotate key to enable 			position.  The roof access security system is armed and the cyclotron can 			now be operated.

Related and Supporting Documentation

•	Utilities Management Program, Engineering Services Policy C-001

•	Safety Reference Cards

•	On Call and Call Back Procedures, Engineering Services Policy A-115